Lab 5 - Neighbor Discovery

Cisco Discovery Protocol (CDP) is a Layer 2 Protocol that is used to discover directly connected cisco devices.

CDP is cisco proprietary protocol and works on all Cisco-manufactured equipment.

CDP will discover directly connected cisco devices and provide details like device Hostname, Device Capability, Platform details, Local interface ID on which the neighbor device is connected, Interface port ID of the neighbor device, Interface IP address of the connected neighbor device, IOS version of the connected neighbor device, Port duplex status of the connected neighbor device, etc.

LLDP (Link Layer Discovery Protocol) is industry standard protocol that can be used for discovering cisco and non-cisco devices. LLDP is disabled by default on Cisco devices and needs to be enabled.

Task 1: Neighbor Discovery using CDP:

Using the neighbor discovery protocol CDP, discover directly connected devices and analyze the output.

Step 1: Access the console of Switch (SW2) and discover directly connected cisco devices using command “show cdp neighbor” and analyze the output:

SW2> enable

SW2# config terminal

SW2(config)# show cdp neighbors

Capability Codes: R - Router, T - Trans Bridge, B - Source Route Bridge

S - Switch, H - Host, I - IGMP, r - Repeater, P - Phone,

D - Remote, C - CVTA, M - Two-port Mac Relay

Device ID        Local Intrfce     Holdtme    Capability     Platform       Port     ID

SW1              Eth 0/0           167         S             Linux Uni      Eth      0/2

PC-02            Eth 0/2           167                       Linux Uni      Eth      0/0

R1               Eth 0/1           132         R             Linux Uni      Eth      0/0

SW2#

The output shows that SW2 switch has 3 directly connected neighbor devices viz SW1, PC-02 and R1. Local interface Eth 0/0 of SW2 is connected to Interface Eth 0/2 of neighbor SW1. Similarly, local interface Eth 0/2 of SW2 is connected to Interface Eth 0/0 of neighbor PC-02 and similarly local interface Eth 0/1 of SW2 is connected to Interface Eth 0/0 of neighbor R1.

It is showing that capability of neighbor device SW1 is S (S – Switch), capability of neighbor device R1 is R (R – Router).

Step 2: Access the console of Switch (SW2) and get details of directly connected cisco devices using command “show cdp neighbor detail” and analyze the output:

SW2> enable

SW2# config terminal

SW2(config)# show cdp neighbors detail

-------------------------

Device ID: SW1

Entry address(es):

IP address: 10.0.0.100

Platform: Linux Unix,  Capabilities: Switch

Interface: Ethernet0/0,  Port ID (outgoing port): Ethernet0/2

Holdtime : 149 sec

Version :

Cisco IOS Software, Solaris Software (I86BI_LINUXL2-IPBASEK9-M), Experimental Version 15.1(20130509:024903) [dstivers-may8-2013-team_track 103]

Copyright (c) 1986-2013 by Cisco Systems, Inc.

Compiled Wed 08-May-13 21:05 by dstivers

advertisement version: 2

VTP Management Domain: ''

Duplex: half

Management address(es):

IP address: 10.0.0.100

-------------------------

Device ID: PC-02

Entry address(es):

IP address: 10.0.0.20

Platform: Linux Unix,  Capabilities:

Interface: Ethernet0/2,  Port ID (outgoing port): Ethernet0/0

Holdtime : 130 sec

Version :

Cisco IOS Software, Linux Software (I86BI_LINUX-ADVENTERPRISEK9-M), Version 15.2(4)M1, DEVELOPMENT TEST SOFTWARE

Technical Support: http://www.cisco.com/techsupport

Copyright (c) 1986-2012 by Cisco Systems, Inc.

Compiled Fri 27-Jul-12 10:57 by prod_rel_team

advertisement version: 2

Duplex: half

Management address(es):

-------------------------

Device ID: R1

Entry address(es):

IP address: 10.0.0.1

Platform: Linux Unix,  Capabilities: Router

Interface: Ethernet0/1,  Port ID (outgoing port): Ethernet0/0

Holdtime : 171 sec

Version :

Cisco IOS Software, Linux Software (I86BI_LINUX-ADVENTERPRISEK9-M), Version 15.2(4)M1, DEVELOPMENT TEST SOFTWARE

Technical Support: http://www.cisco.com/techsupport

Copyright (c) 1986-2012 by Cisco Systems, Inc.

Compiled Fri 27-Jul-12 10:57 by prod_rel_team

advertisement version: 2

Duplex: half

Management address(es):

The output shows details like device Hostname, Device Capability, Platform details, Local interface ID on which the neighbor device is connected, Interface port ID of the neighbor device, Interface IP address of the connected neighbor device, IOS version of the connected neighbor device, Port duplex status of the connected neighbor device, etc. for all neighbors.

Task 2: Enable/Disable Neighbor Discovery:

CDP is by default enabled, CDP can be disabled on per port basis from interface configuration mode or CDP can be disabled globally for all ports from global configuration mode.

Step 1: Access the console of Switch (SW2) and verify if CDP is operational on your device, use following command and verify details:

SW2> enable

SW2# show cdp

Global CDP information:

Sending CDP packets every 60 seconds

Sending a hold time value of 180 seconds

Sending CDPv2 advertisements is enabled

Output shows that CDPv2 is enabled by default and it is sending CDP control messages every 60 seconds and default CDP hold down timer is 180seconds.

Step 2: Access the console of Switch (SW2) and use following command in global configuration mode to disable CDP neighbor discovery for all ports:

SW2# config t

SW2(config)#no cdp run

SW2(config)#exit

SW2#

SW2# show cdp

% CDP is not enabled

Step 3: Access the console of Switch (SW2) and use following command in global configuration mode to enable CDP neighbor discovery for all ports:

SW2# config t

SW2(config)#cdp run

SW2(config)#exit

SW2#

SW2# show cdp

Global CDP information:

Sending CDP packets every 60 seconds

Sending a hold time value of 180 seconds

Sending CDPv2 advertisements is enabled

Step 4: Access the console of Switch (SW2) and use following command in interface configuration mode to disable CDP neighbor discovery for Eth 0/0 port and verify:

SW2(config)# interface ethernet 0/0

SW2(config-if)# no cdp enable

SW2(config-if)# end

SW2# show cdp interface eth 0/0

CDP is not enabled on interface Ethernet0/0

Step 5: Access the console of Switch (SW2) and use following command in interface configuration mode to enable CDP neighbor discovery for Eth 0/0 port and verify:

SW2(config)# interface ethernet 0/0

SW2(config-if)# cdp enable

SW2(config-if)# end

SW2# show cdp interface eth 0/0

Ethernet0/0 is up, line protocol is up

Encapsulation ARPA

Sending CDP packets every 60 seconds

Holdtime is 180 seconds

Task 3: CDP Duplex Mismatch Discovery:

CDP control messages that are exchanged between neighbors every 60 seconds carry information about port duplex, VTP management domain of neighbor devices, CDP-related counters, and VLAN IDs of connecting ports.

Let us change the port duplex of E0/0 port on SW2 Switch and verify if CDP Duplex-Mismatch message is sent to SW1 Switch.

Step 1: Access the console of Switch (SW2) and use following command to change duplex mode of interface Ethernet0/0 to Full duplex:

SW2> enable

SW2# config t

SW2(config)# int ethernet 0/0

SW2(config-if)#duplex full

*Jan  6 14:51:46.236: %CDP-4-DUPLEX_MISMATCH: duplex mismatch discovered on Ethernet0/0 (not half duplex), with SW1 Ethernet0/2 (half duplex).

As seen above, switch will immediately log the system message generated by CDP about the duplex mismatch with SW1.

Let us view the system log on SW1 and verify if the CDP control message carrying information about port duplex is received from neighbor.

Step 2: Access the console of Switch (SW1) and use following command to verify the system log:

SW1> enable

SW1# show logging

Syslog logging: enabled (0 messages dropped, 0 messages rate-limited, 0 flushes, 0 overruns, xml disabled, filtering disabled)

No Active Message Discriminator.

No Inactive Message Discriminator.

Console logging: level debugging, 42 messages logged, xml disabled,

filtering disabled

Monitor logging: level debugging, 0 messages logged, xml disabled,

filtering disabled

Buffer logging:  level debugging, 42 messages logged, xml disabled,

filtering disabled

Exception Logging: size (4096 bytes)

Count and timestamp logging messages: disabled

Persistent logging: disabled

Trap logging: level informational, 45 message lines logged

Logging Source-Interface:       VRF Name:

Log Buffer (4096 bytes):

*Jan  6 14:51:46.257: %CDP-4-DUPLEX_MISMATCH: duplex mismatch discovered on Ethernet0/2 (not full duplex), with SW2 Ethernet0/0 (full duplex).

*Jan  6 14:52:46.257: %CDP-4-DUPLEX_MISMATCH: duplex mismatch discovered on Ethernet0/2 (not full duplex), with SW2 Ethernet0/0 (full duplex).

Step 3: Access the console of Switch (SW2) and use following command to restore the duplex mode to half duplex:

SW2> enable

SW2# config t

SW2(config)# int ethernet 0/0

SW2(config-if)# no duplex full