Lab 7 - Configure Basic IPv4 Connectivity

Task 1: Configure IPv4 Addresses on Branch Routers:

In this task, we will configure IPv4 addresses on all relevant Interface of each branch router (R2, R3) and Head-office router (R1).

LAN Equipment’s like Switches (SW2 and SW3) and all branch computers (PC-02 and PC-03) and Server (SRV) are pre-configured with IP addresses, default-gateways, usernames, passwords, and all other configurations.

Step 1: Access the console of Head-office router R1 and enter interface config mode to configure IPv4 addresses as seen in diagram. Use the following command:

Note: Password for privilege EXEC mode is 'cisco'

R1> enable

R1# config terminal

R1(config)# interface ethernet0/0

R1(config-if)# ip address 10.0.0.1 255.255.255.0

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config-if)# interface serial1/2

R1(config-if)# ip address 12.0.0.1 255.255.255.0

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config-if)# interface serial1/3

R1(config-if)# ip address 13.0.0.1 255.255.255.0

R1(config-if)# no shutdown

R1(config-if)# end

R1#

Step 2: Access the console of R2 Branch-1 router and enter interface config mode to configure IPv4 addresses as seen in diagram. Use the following command:

Note: Password for privilege EXEC mode is 'cisco'

R2> enable

R2# config terminal

R2(config)# interface ethernet0/0

R2(config-if)# ip address 20.0.0.1 255.255.255.0

R2(config-if)# no shutdown

R2(config-if)# exit

R2(config-if)# interface serial1/2

R2(config-if)# ip address 12.0.0.2 255.255.255.0

R2(config-if)# no shutdown

R2(config-if)# exit

R2(config-if)# interface serial1/0

R2(config-if)# ip address 23.0.0.2 255.255.255.0

R2(config-if)# no shutdown

R2(config-if)# end

R2#

Step 3: Access the console of R3 Branch-2 router and enter interface config mode to configure IPv4 addresses as seen in diagram. Use the following command:

Note: Password for privilege EXEC mode is 'cisco'

R3> enable

R3# config terminal

R3(config)# interface ethernet0/0

R3(config-if)# ip address 30.0.0.1 255.255.255.0

R3(config-if)# no shutdown

R3(config-if)# exit

R3(config-if)# interface serial1/3

R3(config-if)# ip address 13.0.0.3 255.255.255.0

R3(config-if)# no shutdown

R3(config-if)# exit

R3(config-if)# interface serial1/0

R3(config-if)# ip address 23.0.0.3 255.255.255.0

R3(config-if)# no shutdown

R3(config-if)# end

R3#

Task 2: Verify Configuration:

Step 1: Access the console of R1 router and enter the Privilege EXEC mode. Verify the IPv4 configuration using following command:

R1# show ip interface brief

Interface              IP-Address      OK?     Method     Status                    Protocol

Ethernet0/0            10.0.0.1        YES     manual     up                        up

Ethernet0/1            unassigned      YES     TFTP       administratively down     down

Ethernet0/2            unassigned      YES     TFTP       administratively down     down

Ethernet0/3            unassigned      YES     TFTP       administratively down     down

Serial1/0              unassigned      YES     TFTP       administratively down     down

Serial1/1              unassigned      YES     TFTP       administratively down     down

Serial1/2              12.0.0.1        YES     manual     up                        up

Serial1/3              13.0.0.1        YES     manual     up                        up

R1#ping 12.0.0.2

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 12.0.0.2, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 8/8/9 ms

R1#ping 13.0.0.3

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 13.0.0.3, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 7/8/9 ms

R1#ping 10.0.0.10

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.0.0.10, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/1 ms

Step 2: Access the console of R2 router and enter the Privilege EXEC mode. Verify the IPv4 configuration using following command:

R2# show ip interface brief

Interface              IP-Address        OK?      Method      Status                    Protocol

Ethernet0/0            20.0.0.1          YES      manual      up                        up

Ethernet0/1            unassigned        YES      TFTP        administratively down     down

Ethernet0/2            unassigned        YES      TFTP        administratively down     down

Ethernet0/3            unassigned        YES      TFTP        administratively down     down

Serial1/0              23.0.0.2          YES      manual      up                        up

Serial1/1              unassigned        YES      TFTP        administratively down     down

Serial1/2              12.0.0.2          YES      manual      up                        up

Serial1/3              unassigned        YES      TFTP        administratively down     down

R2#ping 12.0.0.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 12.0.0.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 8/8/9 ms

R2#ping 23.0.0.3

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 23.0.0.3, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 8/8/8 ms

R2#ping 20.0.0.10

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 20.0.0.10, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/1 ms

Step 3: Access the console of R1 router and enter the Privilege EXEC mode. Verify the IPv4 configuration using following command:

R3# show ip interface brief

Interface             IP-Address      OK?       Method     Status                    Protocol

Ethernet0/0           30.0.0.1        YES       manual     up                        up

Ethernet0/1           unassigned      YES       TFTP       administratively down     down

Ethernet0/2           unassigned      YES       TFTP       administratively down     down

Ethernet0/3           unassigned      YES       TFTP       administratively down     down

Serial1/0             23.0.0.3        YES       manual     up                        up

Serial1/1             unassigned      YES       TFTP       administratively down     down

Serial1/2             unassigned      YES       TFTP       administratively down     down

Serial1/3             13.0.0.3        YES       manual     up                        up

R3#ping 13.0.0.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 13.0.0.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 8/8/9 ms

R3#ping 23.0.0.2

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 23.0.0.2, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 8/8/9 ms

R3#ping 30.0.0.10

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 30.0.0.10, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/1 ms