Lab 9 - Configure and Verify IPv4 Static Routes

Task 1: Configure Default routes on Branch Routers:

In this task, we will configure IPv4 connectivity between branch router (R2, R3) and Head-office router (R1). We will use static routes for inter office reachability.

LAN Equipment’s like Switches (SW2 and SW3) and all branch computers (PC-02 and PC-03) and Server (SRV) are pre-configured with IP addresses, default-gateways, usernames, passwords, and all other configurations.

Step 1: Access the console of R2 Branch-1 router and enter global configuration mode to configure IPv4 static default routes to reach head-office router. refer diagram as required. Use the following command:

Note: Password for privilege EXEC mode is 'cisco'

R2> enable

R2# config terminal

R2(config)# ip route 0.0.0.0 0.0.0.0 12.0.0.1

R2(config)# end

R2#

Step 2: Access the console of R3 Branch-2 router and enter global configuration mode to configure IPv4 static default routes to reach head-office router. Refer diagram as required. Use the following command:

Note: Password for privilege EXEC mode is 'cisco'

R3> enable

R3# config terminal

R3(config)# ip route 0.0.0.0 0.0.0.0 13.0.0.1

R3(config)# end

R3#

Task 2: Configure Static Routes on Head-Office Routers:

In this task, we will configure IPv4 connectivity between Head-office router (R1) and branch office router (R2, R3). We will use static routes for inter office reachability.

Step 1: Access the console of Head-office router R1 and enter global configuration mode to configure static routes to reach Branch office networks. Refer diagram as required. Use the following command:

Note: Password for privilege EXEC mode is 'cisco'

R1> enable

R1# config terminal

R1(config)# ip route 20.0.0.0 255.255.255.0 12.0.0.2

R1(config)# ip route 30.0.0.0 255.255.255.0 13.0.0.3

R1(config)# end

R1#

Task 3: Verify Configuration:

Step 1: Access the console of R1 router and enter the Privilege EXEC mode. Verify the static route configuration using following command:

R1# show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

+ - replicated route, % - next hop override

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        10.0.0.0/24 is directly connected, Ethernet0/0

L        10.0.0.1/32 is directly connected, Ethernet0/0

12.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        12.0.0.0/24 is directly connected, Serial1/2

L        12.0.0.1/32 is directly connected, Serial1/2

13.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        13.0.0.0/24 is directly connected, Serial1/3

L        13.0.0.1/32 is directly connected, Serial1/3

20.0.0.0/24 is subnetted, 1 subnets

S        20.0.0.0 [1/0] via 12.0.0.2

30.0.0.0/24 is subnetted, 1 subnets

S        30.0.0.0 [1/0] via 13.0.0.3

We can see two static routes in routing table of R1 (Head-office) router. there is a static route to reach 20.0.0.0/24 LAN Network of Branch-1 and a static route to reach 30.0.0.0/24 LAN Network of Branch-2.

Step 2: Access the console of R2 router and enter the Privilege EXEC mode. Verify the static default route configuration using following command:

R2# show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

+ - replicated route, % - next hop override

Gateway of last resort is 12.0.0.1 to network 0.0.0.0

S*    0.0.0.0/0 [1/0] via 12.0.0.1

12.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        12.0.0.0/24 is directly connected, Serial1/2

L        12.0.0.2/32 is directly connected, Serial1/2

20.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        20.0.0.0/24 is directly connected, Ethernet0/0

L        20.0.0.1/32 is directly connected, Ethernet0/0

23.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        23.0.0.0/24 is directly connected, Serial1/0

L        23.0.0.2/32 is directly connected, Serial1/0

We can see a static default routes in routing table of R2 (Branch-1) router. R2 Branch-1 router is considering the Head-office router as gateway of last resort and forwarding all traffic for which the routes are not available to R1 (Head-Office Router).

Step 3: Access the console of R3 router and enter the Privilege EXEC mode. Verify the static default route configuration using following command:

R3# show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

+ - replicated route, % - next hop override

Gateway of last resort is 13.0.0.1 to network 0.0.0.0

S*    0.0.0.0/0 [1/0] via 13.0.0.1

13.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        13.0.0.0/24 is directly connected, Serial1/3

L        13.0.0.3/32 is directly connected, Serial1/3

23.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        23.0.0.0/24 is directly connected, Serial1/0

L        23.0.0.3/32 is directly connected, Serial1/0

30.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        30.0.0.0/24 is directly connected, Ethernet0/0

L        30.0.0.1/32 is directly connected, Ethernet0/0

We can see a static default routes in routing table of R3 (Branch-2) router. R3 Branch-2 router is considering the Head-office router as gateway of last resort and forwarding all traffic for which the routes are not available to R1 (Head-Office Router).

Step 4: Access the console of R2 router and use ping and traceroute command to check reachability to Head-office LAN network and Branch-2 LAN Network. Verify using following command:

Note: All Computers and Servers on LAN Network in Head-Office, Branch-1 and Branch-2 offices are configured with default gateways pointing towards their Gateway Router. SRV Server has been configured with R1 router as default gateway. Similarly, PC2 of Branch-1 is configured with R2 router as its default gateway and PC3 of Branch-2 is configured with R3 router as its default gateway.

R2# ping 10.0.0.10

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.0.0.10, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 14/15/16 ms

R2#ping 30.0.0.10

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 30.0.0.10, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 27/28/34 ms

R2# traceroute 30.0.0.10

Type escape sequence to abort.

Tracing the route to 30.0.0.10

VRF info: (vrf in name/id, vrf out name/id)

1 12.0.0.1 13 msec 15 msec 12 msec

2 13.0.0.3 31 msec 31 msec 24 msec

3 30.0.0.10 29 msec *  31 msec

R2#

We can see that the Traceroute traffic from Branch-1 for Remote Branch-2 is going through the head Office Router(12.0.0.1) as R2 the (Branch-1) office router has default gateway configured pointing to the Head-Office router R1. Though we have a direct connection between Branch-1(R2) router and Branch-2(R3) but because we don’t have route configured through this path, so it is not used.

Step 5: Access the console of R3 router and use ping and traceroute command to check reachability to Head-office LAN network and Branch-1 LAN Network. Verify using following command:

R3# ping 10.0.0.10

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.0.0.10, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 14/15/16 ms

R3# ping 20.0.0.10

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 20.0.0.10, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 27/28/34 ms

R3# traceroute 20.0.0.10

Type escape sequence to abort.

Tracing the route to 20.0.0.10

VRF info: (vrf in name/id, vrf out name/id)

1 13.0.0.1 13 msec 15 msec 12 msec

2 12.0.0.2 31 msec 31 msec 24 msec

3 20.0.0.10 29 msec *  31 msec

R3#

We can see that the Traceroute traffic from Branch-2 for Remote Branch-1 is going through the head Office Router(13.0.0.1) as R3 the (Branch-2) office router has default gateway configured pointing to the Head-Office router R1. Though we have a direct connection between Branch-2(R3) router and Branch-1(R2) but because we do not have route configured through this path, so it is not used.

Step 6: Access the console of PC-2 and use ping command to check reachability to Server (SRV) in Head-office LAN network and PC-3 in Branch-2 LAN Network. Verify using following command:

**************************************************************************

**************************************************************************

**                       Welcome to PC-02 MENU                          **

**                                                                      **

**      Press 1 --> To Ping 20.0.0.1 (R2 - Default-gateway)             **

**                                                                      **

**      Press 2 --> To Ping 12.0.0.1 (R1 - Head-office Router)          **

**                                                                      **

**      Press 3 --> To Ping 23.0.0.3 (R3 - Branch-2 Router)             **

**                                                                      **

**      Press 4 --> To Ping 10.0.0.10 (SRV - Head-office Server)        **

**                                                                      **

**      Press 5 --> To Ping 30.0.0.10 (PC3 - Branch-2 PC)               **

**                                                                      **

**      Press 6 --> To Trace 30.0.0.10 (PC3 - Branch-2 PC)              **

**                                                                      **

**      Press q --> To exit the device                                  **

**                                                                      **

**************************************************************************

**************************************************************************

4

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.0.0.10, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 15/15/16 ms

5

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 30.0.0.10, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 30/30/32 ms

6

Type escape sequence to abort.

Tracing the route to 30.0.0.10

VRF info: (vrf in name/id, vrf out name/id)

1 20.0.0.1 0 msec 1 msec 1 msec

2 12.0.0.1 16 msec 20 msec 14 msec

3 13.0.0.3 16 msec 26 msec 27 msec

4 30.0.0.10 30 msec *  22 msec

We can see that the Traceroute traffic from PC-02 for Remote Branch-2 PC-03 is going through the head Office Router(12.0.0.1) as R2 the (Branch-1) office router has default gateway configured pointing to the Head-Office router R1. Though we have a direct connection between Branch-1(R2) router and Branch-2(R3) but because we don’t have route configured through this path, so it is not used.

Task 4: Configure Static Route between R2 and R3:

Step 1: Access the console of R2 router and enter the Global configuration mode. Configure more specific static route to directly reach the LAN Network of Branch-2 using following command:

R2# configure terminal

R2(config)# ip route 30.0.0.0 255.255.255.0 23.0.0.3

R2(config)#

Step 2: Access the console of R2 router and enter the Global configuration mode. Configure more specific static route to directly reach the LAN Network of Branch-1 using following command:

R3# configure terminal

R3(config)# ip route 20.0.0.0 255.255.255.0 23.0.0.2

R3(config)#

Step 3: Access the console of R2 router and verify routing table and check if we have more specific route to reach 30.0.0.0/24 network (LAN of branch-2). enter the Global configuration mode and type following command:

R2# show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

+ - replicated route, % - next hop override

Gateway of last resort is 12.0.0.1 to network 0.0.0.0

S*    0.0.0.0/0 [1/0] via 12.0.0.1

12.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        12.0.0.0/24 is directly connected, Serial1/2

L        12.0.0.2/32 is directly connected, Serial1/2

20.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        20.0.0.0/24 is directly connected, Ethernet0/0

L        20.0.0.1/32 is directly connected, Ethernet0/0

23.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        23.0.0.0/24 is directly connected, Serial1/0

L        23.0.0.2/32 is directly connected, Serial1/0

30.0.0.0/24 is subnetted, 1 subnets

S        30.0.0.0 [1/0] via 23.0.0.3

As seen above in the Routing table now we have a more specific route to reach 30.0.0.0/24 network along with the default route. Router will always forward packets to more specific route and only if specific routes are note available it will use the gateway of last resort the default route.

Step 4: Access the console of R3 router and verify routing table and check if we have more specific route to reach 20.0.0.0/24 network (LAN of branch-1). enter the Global configuration mode and type following command:

R3# show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

+ - replicated route, % - next hop override

Gateway of last resort is 13.0.0.1 to network 0.0.0.0

S*    0.0.0.0/0 [1/0] via 13.0.0.1

13.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        13.0.0.0/24 is directly connected, Serial1/3

L        13.0.0.3/32 is directly connected, Serial1/3

20.0.0.0/24 is subnetted, 1 subnets

S        20.0.0.0 [1/0] via 23.0.0.2

23.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        23.0.0.0/24 is directly connected, Serial1/0

L        23.0.0.3/32 is directly connected, Serial1/0

30.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        30.0.0.0/24 is directly connected, Ethernet0/0

L        30.0.0.1/32 is directly connected, Ethernet0/0

As seen above in the Routing table now we have a more specific route to reach 20.0.0.0/24 network along with the default route. Router will always forward packets to more specific route and only if specific routes are not available it will use the gateway of last resort the default route.

Task 5: Verify and Analyze the traffic path:

Step 1: Access the console of R2 router and enter the Privilege EXEC mode and type the traceroute command to trace the path that traffic takes from R2 Branch-1 router to 30.0.0.10 PC-03 on LAN of Branch-2. use following command:

R2# traceroute 30.0.0.10

Tracing the route to 30.0.0.10

VRF info: (vrf in name/id, vrf out name/id)

1 23.0.0.3 12 msec 16 msec 23 msec

2 30.0.0.10 16 msec *  15 msec

As seen above in the output, traffic is now taking a more specific path via 23.0.0.3 router to reach 30.0.0.0/24 network traffic is not taking the default route that is pointing via R1 router. Router will always forward packets to more specific route and only if specific routes are not available it will use the gateway of last resort the default route.

Let us shut down the link between R2 and R3 routers and analyze the path that traffic takes to reach 30.0.0.0/24 network from R2 router.

Step 2: Access the console of R2 router and shutdown the interface Serial 1/0 the link between R2 and R3. Use following command:

R2# configure terminal

R2(config)# interface serial1/0

R2(config-if)# shutdown

R2(config-if)#

*Feb 11 15:19:15.959: %LINK-5-CHANGED: Interface Serial1/0, changed state to administratively down

*Feb 11 15:19:16.970: %LINEPROTO-5-UPDOWN: Line protocol on Interface Serial1/0, changed state to down

Step 3: Analyze the traffic path using the traceroute command. use following command:

R2# traceroute 30.0.0.10

Type escape sequence to abort.

Tracing the route to 30.0.0.10

VRF info: (vrf in name/id, vrf out name/id)

1 12.0.0.1 15 msec 23 msec 12 msec

2 13.0.0.3 31 msec 31 msec 31 msec

3 30.0.0.10 31 msec *  30 msec

R2#msec

As seen above in the output, because the link is down between R2 and R3 router the traffic is now taking path provided by default route that is pointing via R1 router.

Task 6: Ping Outputs:

Step 1: Access the console of R3 router and shutdown the interface connected to PC-03. use following command:

R3# configure terminal

R3(config)# interface ethernet 0/0

R3(config-if)# shutdown

R3(config-if)# end

Now Let us ping PC-03 from R2 router and Analyze the ping output.

Step 2: Access the console of R2 router and use ping 30.0.0.10 command to check reachability of PC-03. Use following command:

R2# ping 30.0.0.10

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 30.0.0.10, timeout is 2 seconds:

.....

Success rate is 0 percent (0/5)

This ping attempt was not successful. With the current route configuration, the ICMP echoes will reach PC-03. R2 has a default route via R1, and R1 has route configured to reach 30.0.0.0/24 network. However, because the PC-03 is not active on R3 router hence there is no one to provide the ICMP echo replies Therefore, the period (.) characters in the ping output indicate timeouts on the reply.

Now Let us shutdown the interface Serial1/3 on R1 router (Link to R3 router) so that there is no reachability from R1 router to PC-03.

Step 3: Access the console of R1 router and shutdown the interface connected to R3 router. use following command:

R1# configure terminal

R1(config)# interface serial 1/3

R1(config-if)# no shutdown

R1(config-if)# end

As the link between R1 and R3 is down, so R1 cannot reach PC-03. Now if we ping PC-03 from R2 router then the default route configuration on R2 router will send traffic to R1 router but because R1 router is not able to reach PC-03 so it will send U character as ping output.

Step 4: Access the console of R2 router and Ping 30.0.0.10 PC-03. use following command:

R2# ping 30.0.0.10

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 30.0.0.10, timeout is 2 seconds:

U.U.U

Success rate is 0 percent (0/5)

This ping attempt fails for a different reason. The link between R1 and R3 is down and hence the route from R1 to reach PC-03 (30.0.0.10) is down. Now if person on R2 router pings PC03 (30.0.0.10), its default route will forward packets to R1 router. and if R1 receives packets for 30.0.0.10 (PC-03) it must drop the packets that are destined for that IP address as it does not have path to reach this network.

The "U" characters in the ping output indicate that a router in the forwarding path returned ICMP Unreachable messages to PC1.