Lab 12 - Understanding and Creating VLANs

Creation of VLANS on a switch can be done in the following ways:

- VLAN database mode: This mode of creating VLANS is now deprecated and cannot be used on newer models of Cisco Switches. Legacy switch models can be configured in this way.

- Creating VLANS in global Configuration mode: The right way to configure modern Cisco Switches.

- Assigning a Port to an unknown VLAN: If a port in introduced into a VLAN that does not exist in the VLAN database, switch will automatically create this VLAN.

- Using API: VLANS can be created using API like REST API, NETCONF, RESTCONF etc.

Task 1: Analyze default allocation of VLANS:

Step 1: Access the console of Switch SW2 and enter Privilege EXEC mode. Type following Monitoring command to Analyze VLAN Allocations:

SW2# show vlan

VLAN     Name                    Status           Ports

----     ----------------------------------------------------------------------

1        default                 active        Et0/0, Et0/1, Et0/2, Et0/3

Et1/0, Et1/1, Et1/2, Et1/3

1002     fddi-default            act/unsup

1003     token-ring-default      act/unsup

1004     fddinet-default         act/unsup

1005     trnet-default           act/unsup

As seen in the output above, Default VLAN 1 and Native VLANs 1002-1005 are automatically created on Switch. These are default VLANs and hence cannot be modified or deleted. VLANs 1002-1005 are Native VLAN for obsolete Technologies such as FDDI and Token-ring that are no more supported in the industry. As seen in the output their status is be displayed as “unsup” which mean unsupported. By default, all Ports on a Cisco Switch will be member of VLAN 1 unless modified by an administrator.

Task 2: Create VLANS in Global Configuration Mode:

Step 1: Access the console of Switch SW2 and enter Global configuration mode and create the VLAN 10 into the database and assign a name to this VLAN. Type following command:

SW2# configure terminal

SW2(config)# vlan 10

SW2(config-vlan)# name VLAN10

SW2(config-vlan)# end

Step 2: Verify that the newly created VLAN 10 is reflecting in the VLAN Database. Access the console of Switch SW2 and enter Privilege EXEC mode. Type following command to analyze VLAN database:

SW2# Show vlan

VLAN	Name                             	Status		  Ports

---- 	-------------------------------- 	--------- 	-----------------------------

1    	default                          	active    	Et0/0, Et0/1, Et0/2, Et0/3

Et1/0, Et1/1, Et1/2, Et1/3

10   	VLAN10                           	active

1002 	fddi-default                     	act/unsup

1003 	token-ring-default               	act/unsup

1004 	fddinet-default                  	act/unsup

1005 	trnet-default                    	act/unsup

As seen in the output above, VLAN 10 is created in VLAN database of the Switch and this VLAN is active, but no ports are assigned to this VLAN.

Let us Assign Port membership to VLAN 10.

Step 3: Access the console of Switch SW2 and enter Interface configuration mode and assign port Ethernet 0/2 & Ethernet 0/3 as members of VLAN 10. Type following command to achieve this:

SW2# configure terminal

SW2(config)# interface ethernet 0/2

SW2(config-if)# switchport mode access

SW2(config-if)# switchport access vlan 10

SW2(config-if)# exit

SW2(config)# interface ethernet 0/3

SW2(config-if)# switchport mode access

SW2(config-if)# switchport access vlan 10

SW2(config-if)# end

Step 4: Verify that the newly Assigned Port membership to VLAN 10 is reflecting in the VLAN Database. Access the console of Switch SW2 and enter Privilege EXEC mode. Type following command to analyze VLAN database:

SW2# show vlan

VLAN	Name                             	Status		Ports

---- 	-------------------------------- 	--------- 	-----------------------------

1    	default                          	active		Et0/0, Et0/1, Et0/2, Et0/3

Et1/2, Et1/3

10   	VLAN10                           	active    	Et0/2, Et0/3

1002 	fddi-default                     	act/unsup

1003 	token-ring-default               	act/unsup

1004 	fddinet-default                  	act/unsup

In the output above, we can see that ports Ethernet 0/2 and Ethernet 0/3 which were earlier members of VLAN 1 are now members of VLAN 10. A port can be member of only one Data VLAN. So, when we assigned these ports as members of VLAN 10 they moved out of VLAN 1 and became members of VLAN 10.

Task 3: Ping test between PC3 & PC4

As seen in Diagram, PC3 and PC4 both are connected to Switch SW2 on ports Ethernet0/2 and port Ethernet0/3, respectively. Now because these ports E0/2 and E0/3 on SW2 are in same VLAN10 hence communication between should happen. Switch allows intra VLAN Switching.

Let us verify if these PCs can communicate with one another.

NOTE: IPv4 addresses for PC3 (10.0.0.30) and PC4(10.0.0.40) are pre-configured. Both these PCs are in the same subnet 10.0.0.0/8. Which means, if both are configured within the same VLAN, they should communicate with one another.

Step 1: Access the console of PC3 and select option from menu to ping PC4:

**************************************************************************

**************************************************************************

**                       Welcome to PC-03 MENU                          **

**                                                                      **

**      Press 1 --> To Ping 10.0.0.10 (PC1)                             **

**                                                                      **

**      Press 2 --> To Ping 10.0.0.20 (PC2)                             **

**                                                                      **

**      Press 3 --> To Ping 10.0.0.40 (PC4)                             **

**                                                                      **

**      Press 4 --> To Ping 10.0.0.100 (SW1 - Management IP)            **

**                                                                      **

**      Press 5 --> To Ping 10.0.0.200 (SW2 - Management IP)            **

**                                                                      **

**      Press q --> To exit the device                                  **

**************************************************************************

**************************************************************************

3

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.0.0.40, timeout is 2 seconds:

.!!!!

Success rate is 80 percent (4/5), round-trip min/avg/max = 1/1/2 ms

In the output above, we can see that from PC-03 we are able to ping PC4 that is because both these PCs are in Same subnet and they are members of same VLAN 10.

Task 4: Creation of VLAN by Port Assignment

We will automatically create a VLAN by assigning a Port to it. Let us assign port Ethernet 0/3 to VLAN 20 without creating it first.

Step 1: Access the console of SW2 and in interface configuration mode assign VLAN membership to interface Ethernet0/3. Use following command to achieve this:

SW2# configure terminal

SW2(config)# interface ethernet 0/3

SW2(config-if)# switchport mode access

SW2(config-if)# switchport access vlan 20

% Access VLAN does not exist. Creating vlan 20

Sw1(config-if)#end

As seen in the output when we configured interface Ethernet0/3 as member of VLAN 20. Switch found that this VLAN does not exist in VLAN database, so it immediately creates one and assigns the Vlan membership to the port. now the interface ethernet0/3 that was earlier member of VLAN10 is assigned membership of VLAN20.

Let us verify if VLAN 20 got created in VLAN database.

Step 2: Access the console of SW2 and in Privilege EXEC mode verify creation of VLAN 20 using following command:

SW2# show vlan

VLAN	Name                             	Status		Ports

---- 	-------------------------------- 	--------- 	-----------------------------

1    	default                          	active		Et0/0, Et0/1, Et1/0, Et1/1

Et1/2, Et1/3

10   	VLAN10                           	active    	Et0/2,

20   	VLAN0020                          active    	Et0/3,

1002 	fddi-default                     	act/unsup

1003 	token-ring-default               	act/unsup

1004 	fddinet-default                  	act/unsup

1005 	trnet-default                    	act/unsup

As seen in the output of show VLAN on SW2, SW2 has automatically created VLAN 20 and a name VLANXXXX (XXXX denote the 4-digit VLAN number) was assigned.

Also note that now the interface Ethernet 0/3 is not a member of VLAN 10 and it is now a member of VLAN 20 hence the PC-03 and PC-04 connected to these ports are now in different VLANs and hence they do not have reachability to each other.

Let us verify if these PCs can communicate with one another.

Step 3: Access the console of PC3 and select option from menu to ping PC4:

**************************************************************************

**************************************************************************

**                       Welcome to PC-03 MENU                          **

**                                                                      **

**      Press 1 --> To Ping 10.0.0.10 (PC1)                             **

**                                                                      **

**      Press 2 --> To Ping 10.0.0.20 (PC2)                             **

**                                                                      **

**      Press 3 --> To Ping 10.0.0.40 (PC4)                             **

**                                                                      **

**      Press 4 --> To Ping 10.0.0.100 (SW1 - Management IP)            **

**                                                                      **

**      Press 5 --> To Ping 10.0.0.200 (SW2 - Management IP)            **

**                                                                      **

**      Press q --> To exit the device                                  **

**************************************************************************

**************************************************************************

3

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.0.0.40, timeout is 2 seconds:

.....

Success rate is 0 percent (0/5)

In the output above, we can see that PC-03 is not able to ping PC4 that is because both these PCs are now in different VLANS and traffic will not switch between different VLAN.

Task 5: Assign Management IP Address to VLAN10:

In this section, let us assign a management IP address for VLAN 10 on switch SW2. This IP address can be reached by all computers in VLAN10:

Step 1: Access the console of SW2 and in Interface configuration mode configure IP address to VLAN10 Use Following command to achieve this:

SW2# configure terminal

SW2(config)# interface vlan 10

SW2(config-if)# ip address 10.0.0.200 255.255.255.0

SW2(config-if)# no shutdown

SW2(config-if)# end

Let us verify if PC3 which is member of VLAN 10 can reach the VLAN10 Management IP of Switch SW2.

Step 2: Access the console of PC3 and select option from menu to ping Management IP of switch SW2:

**************************************************************************

**************************************************************************

**                       Welcome to PC-03 MENU                          **

**                                                                      **

**      Press 1 --> To Ping 10.0.0.10 (PC1)                             **

**                                                                      **

**      Press 2 --> To Ping 10.0.0.20 (PC2)                             **

**                                                                      **

**      Press 3 --> To Ping 10.0.0.40 (PC4)                             **

**                                                                      **

**      Press 4 --> To Ping 10.0.0.100 (SW1 - Management IP)            **

**                                                                      **

**      Press 5 --> To Ping 10.0.0.200 (SW2 - Management IP)            **

**                                                                      **

**      Press q --> To exit the device                                  **

**************************************************************************

**************************************************************************

5

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.0.0.200, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/1 ms

In the output above, we can see that PC-03 member of VLAN 10 can ping the VLAN10 IP address of switch SW2. But now the PC4 that is in VLAN 20 wont be able to ping this management IP of VLAN 10. as they are in different VLANs

Let us verify if PC4 which is member of VLAN 20 can reach the VLAN10 Management IP of Switch SW2.

Step 3: Access the console of PC4 and select option from menu to ping Management IP of switch SW2:

**************************************************************************

**************************************************************************

**                       Welcome to PC-04 MENU                          **

**                                                                      **

**      Press 1 --> To Ping 10.0.0.10 (PC1)                             **

**                                                                      **

**      Press 2 --> To Ping 10.0.0.20 (PC2)                             **

**                                                                      **

**      Press 3 --> To Ping 10.0.0.30 (PC3)                             **

**                                                                      **

**      Press 4 --> To Ping 10.0.0.100 (SW1 - Management IP)            **

**                                                                      **

**      Press 5 --> To Ping 10.0.0.200 (SW2 - Management IP)            **

**                                                                      **

**      Press q --> To exit the device                                  **

**************************************************************************

**************************************************************************

5

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.0.0.200, timeout is 2 seconds:

.....

Success rate is 0 percent (0/5)

In the output above, we can see that PC-04 member of VLAN 20 cannot ping the VLAN10 IP address of switch SW2.