Lab 15 - L3-Switch Inter-VLAN Routing

VLANs divide Large LAN into smaller Virtual LANs. Each VLAN is a separate broadcast domain. Whenever hosts in one VLAN need to communicate with hosts in another VLAN, the traffic must be routed between them. This is known as inter-VLAN routing.

Inter-VLAN Routing can be done in following ways:

Single External Router with Multiple Interface:
In this type of Inter-VLAN routing, all VLANs have a dedicated separate router interface. Even though this is the simplest way to perform Inter-VLAN routing, it is not the most efficient way. Having a dedicated router interface for each VLAN, may not be suitable for a network with many VLANs as we may run out of interfaces on the router itself.

Single External Router with Sub-interfaces for every VLAN:
In this type of Inter-VLAN routing, all VLANs are routed via a single router interface. A physical interface is broken down into multiple sub-interfaces. Each sub-interface is then assigned to a VLAN. This is the most efficient way to route between VLANs if hosts are connected to a Layer 2 Switch.

Layer-3 Switch using SVIs (Software Virtual Interface):
In this type of Inter-VLAN routing, all VLANS are routed via the L3-Switch to which the computers are connected. No Separate External Layer 3 device is required in this scenario. On Layer 3 Catalyst switches this type of Inter-VLAN Switching is accomplished by the creation of Layer 3 interfaces (switch virtual interfaces (SVIs) ).

Task 1: Configure L3 Switch for Inter-VLAN Routing:

Here we are going to use SVI (Software Virtual Interfaces) of L3-Switch for Inter-VLAN routing. Our LAB topology has two PC connected to a Switch SW2 and both these PCs are in different VLANs. For Inter-VLAN communication we going to configure Svi for each VLAN and enable L3 routing on Switch. Configurations to be done as mentioned below.

| Device | Port Number | Configuration |
| --- | --- | --- |
| SW2 | Ethernet 0/2 | VLAN 10 |
| SW2 | Ethernet 0/3 | VLAN 20 |
| SVI on SW2 | Interface VLAN 10 | IP address 10.0.0.1 255.255.255.0 |
| SVI on SW2 | Interface VLAN 20 | IP address 20.0.0.1 255.255.255.0 |

Note: PCs are pre-configured with IP addresses and default gateways as shown in Diagram. PC3(10.0.0.30) and PC2(10.0.0.20) are members of VLAN 10 and configured with IP address of subnet 10.0.0.0/24. PC1(20.0.0.10) and PC4(20.0.0.40) are members of VLAN 20 and configured with IP address of subnet 20.0.0.0/24.

Step 1: Access the console of Switch SW2 and enter Global Configuration mode. Type following command to assign ports to relevant VLANs and create SVIs and enable routing:

SW2# configure terminal

SW2(config)# ip routing

SW2(config)# interface ethernet 0/2

SW2(config-if)# switchport mode access

SW2(config-if)# switchport access vlan 10

SW2(config-if)# no shutdown

SW2(config-if)# exit

SW2(config)# interface ethernet 0/3

SW2(config-if)# switchport mode access

SW2(config-if)# switchport access vlan 20

SW2(config-if)# no shutdown

SW2(config-if)# exit

SW2(config)# interface vlan 10

SW2(config-if)# ip address 10.0.0.1 255.255.255.0

SW2(config-if)# no shutdown

SW2(config-if)# exit

SW2(config)# interface vlan 20

SW2(config-if)# ip address 20.0.0.1 255.255.255.0

SW2(config-if)# no shutdown

SW2(config-if)# end

As seen above we have configured Interface Ethernet 0/2 to be member of VLAN 10 and similarly, we have configured interface Ethernet0/3 to be member of VLAN 20.

We have configured Software Virtual Interface (SVI) for VLAN 10 and assigned IP address 10.0.0.1/24. Similarly, we have configured Software Virtual Interface (SVI) for VLAN 20 and assigned IP address 20.0.0.1/24.

Now let us verify if Inter-VLAN routing is happening through the SVIs and PC3(VLAN10) can communicate with PC4(VLAN20).

Task 2: Verification:

Let us verify if the inter-VLAN communication is happening with help of SVI on L3-Switch and PC3(VLAN10) can communicate with PC4(VLAN20).

Step 1: Access the console of PC4 (VLAN20) having IP address 20.0.0.40 and ping PC3 (VLAN 10) having IP address 10.0.030 and verify Inter-VLAN communication is happening:

**************************************************************************

**************************************************************************

**                       Welcome to PC-04 MENU                          **

**                                                                      **

**      Press 1 --> To Ping 20.0.0.10 (PC1)                             **

**                                                                      **

**      Press 2 --> To Ping 10.0.0.20 (PC2)                             **

**                                                                      **

**      Press 3 --> To Ping 10.0.0.30 (PC3)                             **

**                                                                      **

**      Press 4 --> To Ping 10.0.0.1 (R1 - VLAN 10 Sub-Interface)       **

**                                                                      **

**      Press 5 --> To Ping 20.0.0.1 (R1 - VLAN 20 Sub-Interface)       **

**                                                                      **

**      Press q --> To exit the device                                  **

**************************************************************************

**************************************************************************

3

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.0.0.30, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/2 ms

The output above shows that from PC4(VLAN 20) having IP address 20.0.0.40, we can ping PC3 (VLAN 10) having IP address 10.0.0.30. our L3-Switch SVI configuration is working.

We can further configure Trunk between SW1 and SW2 and Ping from PC1(VLAN 20) with other PCs in any VLAN.

Task 3: Configure Trunk and VLANs on SW1:

Step 1: Access the console of SW1 and enter interface configuration mode. Configure E0/0 as member of VLAN 10, E0/1 as member of VLAN 20 and E0/2 as Trunk with encapsulation dot1q. Use following command:

SW1# configure terminal

SW1(config)# interface ethernet0/0

SW1(config-if)# switchport mode access

SW1(config-if)# switchport access vlan 10

SW1(config-if)# no shutdown

SW1(config-if)# exit

SW1(config)# interface ethernet 0/1

SW1(config-if)# switchport mode access

SW1(config-if)# switchport access vlan 20

SW1(config-if)# no shutdown

SW1(config-if)# exit

SW1(config-if)# interface ethernet 0/2

SW1(config-if)# switchport trunk encapsulation dot1q

SW1(config-if)# switchport mode trunk

SW1(config-if)# end

As seen above we have configured VLAN10 on Ethernet 0/0 port, this is the port on which PC2(10.0.0.20) is connected. we have configured VLAN20 on Ethernet 0/1 port, this is the port on which PC1(20.0.0.10) is connected and we have configured Trunk on interface Ethernet 0/2, this is the port connected to SW2.

Now let us also configure Trunk on SW2 with default encapsulation of dot1q.

Step 2: Access the console of SW2 and enter interface configuration mode. Configure E0/0 as Trunk with encapsulation dot1q. Use following command:

SW2# configure terminal

SW2(config)# interface ethernet0/0

SW2(config-if)# switchport trunk encapsulation dot1q

SW2(config-if)# switchport mode trunk

SW2(config-if)# no shutdown

SW2(config-if)# end

As seen above we have configured Trunk on interface Ethernet 0/0, this is the port connected to SW1.

Now AS all configurations are in place all PCs should be able to communicate with all other PCs. Communication within VLAN will happen directly as switch switches within VLAN and for Inter-VLAN communication the SVIs on L3-Switch SW2 will be used to route between VLANs.

Let us Verify That all computers can now communicate with all other computers.

Task 4: Verification:

Step 1: Access the console of PC1 (VLAN20) having IP address 20.0.0.10 and ping all other PCs and verify Inter-VLAN communication is happening:

**************************************************************************

**************************************************************************

**                       Welcome to PC-01 MENU                          **

**                                                                      **

**      Press 1 --> To Ping 10.0.0.20 (PC2)                             **

**                                                                      **

**      Press 2 --> To Ping 10.0.0.30 (PC3)                             **

**                                                                      **

**      Press 3 --> To Ping 20.0.0.40 (PC4)                             **

**                                                                      **

**      Press 4 --> To Ping 10.0.0.1 (R1 - VLAN 10 Sub-Interface)       **

**                                                                      **

**      Press 5 --> To Ping 20.0.0.1 (R1 - VLAN 20 Sub-Interface)       **

**                                                                      **

**      Press q --> To exit the device                                  **

**************************************************************************

**************************************************************************

1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.0.0.20, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 2/2/3 ms

2

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.0.0.30, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/2 ms

3

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 20.0.0.40, timeout is 2 seconds:

.!!!!

Success rate is 80 percent (4/5), round-trip min/avg/max = 2/2/3 ms

As seen in the output above PC1 can communicate with all other PCs in all VLANs.