Lab 17 - EtherChannel

EtherChannel: EtherChannel is a port link aggregation technology in which multiple physical port links are grouped into one logical link. It is used to provide high speed links and redundancy. Maximum of 8 links can be aggregated to form a single logical link. EtherChannel combines multiple parallel Links of equal speed between the same pair of switches. The switches treat the EtherChannel port as a single interface, if one of the links fails, but at least one of the links is up, Ether-Channel will remain up.

But, when we connect the switches with more than one link, STP (Spanning Tree Protocol) will block all the links and only keep one active link. That is where EtherChannel comes into picture, all the links (that are grouped as one logical link) will be treated as single logical link therefore no link will be blocked and, it will provide us high speed link and redundancy in our network.

To form an EtherChannel, all ports should have:

- Same duplex

- Same speed

- Same VLAN configuration (i.e., native VLAN and allowed VLAN should be same)

- Switch port modes should be same (access or trunk mode)

EtherChannel can be formed in either of the two ways:

- Dynamic Layer 2 EtherChannel: EtherChannel is negotiated and formed automatically. We can use either PAgP (Cisco Proprietary) or LACP (IEEE 802.3ad) to negotiate the Etherchannel.

- Static Layer 2 EtherChannel: EtherChannel is statically designated as a bundle regardless of whether the opposite end has been configured or not. No protocol is used in the formation of such a EtherChannel and hence sometimes also referred to as a protocol-less EtherChannel.

EtherChannel protocols: To form an EtherChannel, there are 2 protocols, port aggregation protocol (PAgP) and link aggregation control protocol (LACP).

Port Aggregation Protocol (PAgP): Port Aggregation Protocol is a Cisco proprietary protocol used to form an EtherChannel.

There are different modes in which you can configure your interface. These are namely:

- ON: In this mode, the interface will be a part of EtherChannel, but no negotiation takes place.

- Desirable: In this mode, the interface will continuously attempt to convert other side interface into an EtherChannel.

- Auto: In this mode, the interface will become a part of EtherChannel if and only of it is requested by the opposite interface.

Link aggregation control protocol (LACP): LACP is standard based protocol (IEEE 802.3ad).

There are different modes in which you can configure your interface. These are namely:

- Active: In this mode the port initiates negotiations with remote ports by sending LACP packets.

- Passive: In this mode the port responds to LACP packets it receives but does not initiate LACP negotiation. In this mode, the port channel group attaches the interface to the bundle.

Task 1: Configure EtherChannel using PAgP Protocol:

In our Lab Scenario we have two switches SW1 and SW2 interconnected using two links connected. Interface E0/2 of SW1 is connected to Interface E0/0 of SW2 and Interface E1/0 of SW1 is connected to Interface E1/0 of SW2. All other ports are in purpose kept in shutdown mode. Let us configure EtherChannel to aggregate these physical port links and group them into one single logical link using PAgP protocol. Let us configure Trunk over the aggregated Port-channel link

Current Switch connectivity and details are as shown below in the table:

| Switch SW1 | Port Number | Switch SW2 | Port Number |
| --- | --- | --- | --- |
| MAC - aabb.cc00.0500 | Ethernet 0/2 | MAC - aabb.cc00.0600 | Ethernet 0/0 |
|  | Ethernet 1/0 |  | Ethernet 1/0 |

Step 1: Access the console of Switch SW1 and enter Interface configuration mode. Type following command to create EtherChannel using PAgP Protocol. We will configure SW1 in auto mode and SW2 in desirable mode also make the Port-channel Interface as Trunk:

SW1# configure terminal

SW1(config)# interface range E0/2, E1/0

SW1(config-if-range)# switchport trunk encapsulation dot1q

SW1(config-if-range)# switchport mode trunk

SW1(config-if-range) # channel-group 1 mode auto

Similarly Let us create EtherChannel using PAgP Protocol on SW2 and select mode desirable. Port-channel Interface should be configured as Trunk:

SW2# configure terminal

SW2(config)# interface range E0/0, E1/0

SW2(config-if-range)# switchport trunk encapsulation dot1q

SW2(config-if-range)# switchport mode trunk

SW2(config-if-range) # channel-group 1 mode desirable

Now let us verify if EtherChannel is configured with PAgP mode.

Step 2: Access the console of Switch SW1 and enter Privilege configuration mode. Type following command to Verify EtherChannel:

SW1# show EtherChannel summary

Flags:  D – down        P – bundled in port-channel

I – stand-alone s – suspended

H – Hot-standby (LACP only)

R – Layer3      S – Layer2

U – in use      f – failed to allocate aggregator

M – not in use, minimum links not met

u – unsuitable for bundling

w – waiting to be aggregated

d – default port

Number of channel-groups in use: 1

Number of aggregators:           1

Group  Port-channel  Protocol    Ports

------+-------------+-----------+-----------------------------------------------

1      Po1(SU)         PagP      Et0/2(P)    Et1/0(P)

Let us also verify EtherChannel Status on SW2

Step 3: Access the console of Switch SW2 and enter Privilege configuration mode. Type following command to Verify EtherChannel:

SW2# show EtherChannel summary

Flags:  D - down        P - bundled in port-channel

I - stand-alone s - suspended

H - Hot-standby (LACP only)

R - Layer3      S - Layer2

U - in use      f - failed to allocate aggregator

M - not in use, minimum links not met

u - unsuitable for bundling

w - waiting to be aggregated

d - default port

Number of channel-groups in use: 1

Number of aggregators:           1

Group  Port-channel  Protocol    Ports

------+-------------+-----------+-----------------------------------------------

1      Po1(SU)         PAgP      Et0/0(P)    Et1/0(P)

As seen above in the output of SW2, EtherChannel has aggregated ports Eth0/0 and Eth1/0 and created port-channel interface Po1 and PAgP Protocol is used to form EtherChannel on both Switches. As seen Channel-Group 1 is active with Port-channel interface with Flag (SU) where S=Layer2 and U=in Use. The flag indicates that the EtherChannel is operational in Layer 2 mode. Both Ports have Flag (P) which indicates that ports are bundled in port-channel.

Task 2: Configure EtherChannel using LACP Protocol:

In our Lab Scenario we have two switches SW1 and SW2 interconnected using two links connected. Interface E0/2 of SW1 is connected to Interface E0/0 of SW2 and Interface E1/0 of SW1 is connected to Interface E1/0 of SW2. All other ports are in purpose kept in shutdown mode. Let us configure EtherChannel to aggregate these physical port links and group them into one single logical link using LACP protocol. Let us configure Trunk over the aggregated Port-channel link

Current Switch connectivity and details are as shown below in the table:

| Switch SW1 | Port Number | Switch SW2 | Port Number |
| --- | --- | --- | --- |
| MAC - aabb.cc00.0500 | Ethernet 0/2 | MAC - aabb.cc00.0600 | Ethernet 0/0 |
|  | Ethernet 1/0 |  | Ethernet 1/0 |

Step 1: Access the console of Switch SW1 and enter Interface configuration mode. Type following command to create EtherChannel using LACP Protocol. We will configure SW1 in Passive mode and SW2 in Active mode also make the Port-channel Interface as Trunk:

SW1# configure terminal

SW1(config)# interface range E0/2, E1/0

SW1(config-if-range)# no channel-group 1

SW1(config-if-range)# switchport trunk encapsulation dot1q

SW1(config-if-range)# switchport mode trunk

SW1(config-if-range)# channel-group 1 mode passive

Similarly Let us create EtherChannel using LACP Protocol on SW2 and select mode Active. Port-channel Interface should be configured as Trunk:

SW2# configure terminal

SW2(config)# interface range E0/0, E1/0

SW2(config-if-range)# no channel-group 1

SW2(config-if-range)# switchport trunk encapsulation dot1q

SW2(config-if-range)# switchport mode trunk

SW2(config-if-range) # channel-group 1 mode Active

Now let us verify if EtherChannel is configured with LACP mode.

Step 2: Access the console of Switch SW1 and enter Privilege configuration mode. Type following command to Verify EtherChannel:

SW1# show EtherChannel summary

Flags:  D - down        P - bundled in port-channel

I - stand-alone s - suspended

H - Hot-standby (LACP only)

R - Layer3      S - Layer2

U - in use      f - failed to allocate aggregator

M - not in use, minimum links not met

u - unsuitable for bundling

w - waiting to be aggregated

d - default port

Number of channel-groups in use: 1

Number of aggregators:           1

Group  Port-channel  Protocol    Ports

------+-------------+-----------+-----------------------------------------------

1      Po1(SU)         LACP      Et0/2(P)    Et1/0(P)

Let us also verify EtherChannel Status on SW2

Step 3: Access the console of Switch SW2 and enter Privilege configuration mode. Type following command to Verify EtherChannel:

SW2# show EtherChannel summary

Flags:  D - down        P - bundled in port-channel

I - stand-alone s - suspended

H - Hot-standby (LACP only)

R - Layer3      S - Layer2

U - in use      f - failed to allocate aggregator

M - not in use, minimum links not met

u - unsuitable for bundling

w - waiting to be aggregated

d - default port

Number of channel-groups in use: 1

Number of aggregators:           1

Group  Port-channel  Protocol    Ports

------+-------------+-----------+-----------------------------------------------

1      Po1(SU)         LACP      Et0/0(P)    Et1/0(P)

As seen above in the output of SW2, EtherChannel has aggregated ports Eth0/0 and Eth1/0 and created port-channel interface Po1 and LACP Protocol is used to form EtherChannel on both Switches. As seen Channel-Group 1 is active with Port-channel interface with Flag (SU) where S=Layer2 and U=in Use. The flag indicates that the EtherChannel is operational in Layer 2 mode. Both Ports have Flag (P) which indicates that ports are bundled in port-channel.

Task 3: Configure Protocol-Less EtherChannel:

In our Lab Scenario we have two switches SW1 and SW2 interconnected using two links connected. Interface E0/2 of SW1 is connected to Interface E0/0 of SW2 and Interface E1/0 of SW1 is connected to Interface E1/0 of SW2. All other ports are in purpose kept in shutdown mode. Let us configure EtherChannel to aggregate these physical port links and group them into one single logical link using Protocol-less configuration. Let us configure Trunk over the aggregated Port-channel link.

Step 1: Access the console of Switch SW1 and enter Interface configuration mode. Type following command to create Protocol-Less EtherChannel. We will configure SW1 and SW2 both in On mode and also make the Port-channel Interface as Trunk:

SW1# configure terminal

SW1(config)# interface range E0/2, E1/0

SW1(config-if-range)# no channel-group 1

SW1(config-if-range)# switchport trunk encapsulation dot1q

SW1(config-if-range)# switchport mode trunk

SW1(config-if-range)# channel-group 1 mode on

Similarly Let us create EtherChannel using LACP Protocol on SW2 and select mode Active. Port-channel Interface should be configured as Trunk:

SW2# configure terminal

SW2(config)# interface range E0/0, E1/0

SW2(config-if-range)# no channel-group 1

SW2(config-if-range)# switchport trunk encapsulation dot1q

SW2(config-if-range)# switchport mode trunk

SW2(config-if-range) # channel-group 1 mode on

Now let us verify if EtherChannel is configured with ON mode.

Step 2: Access the console of Switch SW1 and enter Privilege configuration mode. Type following command to Verify EtherChannel:

SW1# show EtherChannel summary

Flags:  D - down        P - bundled in port-channel

I - stand-alone s - suspended

H - Hot-standby (LACP only)

R - Layer3      S - Layer2

U - in use      f - failed to allocate aggregator

M - not in use, minimum links not met

u - unsuitable for bundling

w - waiting to be aggregated

d - default port

Number of channel-groups in use: 1

Number of aggregators:           1

Group  Port-channel  Protocol    Ports

------+-------------+-----------+-----------------------------------------------

1      Po1(SU)          -        Et0/2(P)    Et1/0(P)

Let us also verify EtherChannel Status on SW2

Step 3: Access the console of Switch SW2 and enter Privilege configuration mode. Type following command to Verify EtherChannel:

SW2# show EtherChannel summary

Flags:  D - down        P - bundled in port-channel

I - stand-alone s - suspended

H - Hot-standby (LACP only)

R - Layer3      S - Layer2

U - in use      f - failed to allocate aggregator

M - not in use, minimum links not met

u - unsuitable for bundling

w - waiting to be aggregated

d - default port

Number of channel-groups in use: 1

Number of aggregators:           1

Group  Port-channel  Protocol    Ports

------+-------------+-----------+-----------------------------------------------

1      Po1(SU)          -        Et0/0(P)    Et1/0(P)

As seen above in the output of SW2, EtherChannel has aggregated ports Eth0/0 and Eth1/0 and created port-channel interface Po1 and it is Protocol-less configuration on both Switches. As seen Channel-Group 1 is active with Port-channel interface with Flag (SU) where S=Layer2 and U=in Use. The flag indicates that the EtherChannel is operational in Layer 2 mode. Both Ports have Flag (P) which indicates that ports are bundled in port-channel.