Lab 32 - Site-To-Site VPN

Configure Site-To-Site VPN

VPN stands for Virtual Private Network. VPN has become the mandatory feature in any organization as it provides private communication over low-costed public infrastructure like internet. In Site-to-Site IPSEC VPN the routers or firewall in a branch, will performs every task like encapsulation/de-encapsulation, tagging/un-tagging as well as encryption/decryption.

Today we are going to perform Site to Site VPN lab between two of our branch routers R1 and R3. We will be simulating the service provider network by using R2 router as the ISP providing internet connectivity for our branch Routers.

Task 1: Configure IP address and verify reachability

Step 1: Configure IP address and default route on R1, R2 and R3 router

R1#conf t

R1(conf)#int gi0/0

R1(conf-if)#ip address 11.11.11.1 255.255.255.0

R1(conf-if)#no shut

R1(conf-if)#int lo 100

R1(conf-if)#ip address 100.0.0.1 255.255.255.0

R1(conf-if)#ip route 0.0.0.0 0.0.0.0 11.11.11.2

--------------------------------------------------

R2#conf t

R2(conf)#int gi0/0

R2(conf-if)#ip address 11.11.11.2 255.255.255.0

R2(conf-if)#no shut

R2(conf-if)#int gi0/1

R2(conf-if)#ip address 22.22.22.2 255.255.255.0

R2(conf-if)#no shut

R2(conf-if)#exit

R2(conf)#ip route 100.0.0.0 255.255.255.0 11.11.11.1

R2(conf)#ip route 200.0.0.0 255.255.255.0 22.22.22.3

--------------------------------------------------

R3#conf t

R3(conf)#int gi0/0

R3(conf-if)#ip address 22.22.22.3 255.255.255.0

R3(conf-if)#no shut

R3(conf-if)#int lo 200

R3(conf-if)#ip address 200.0.0.3 255.255.255.0

R3(conf-if)#ip route 0.0.0.0 0.0.0.0 22.22.22.2

Step 2: Verify end to end reachability from R1 to R3

R1#ping 200.0.0.3

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 200.0.0.3, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 6/8/13 ms

R1#

-------------------------------------------------------

R3#ping 100.0.0.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 100.0.0.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 6/8/13 ms

R3#

Task 2: Configure IPsec Tunnel

To configure an IPSEC Tunnel we will start by building IKE Phase-I, which is the ISAKMP configuration, where we must specify the policy that routers will be using to form the security association. The protocol used to build security association has to be same for both the routers including the policy number.

After configuration of Phase-I, we will then start by building an access-list that will be used to match the exact packet for which router will perform tunneling, encapsulation and encryption. Once that is done then we will configure the Phase-II, which is Transform Set. This Transform Set will be doing all encryption/decryption, encapsulation/de-encapsulation, tagging/un-tagging of all the traffic.

Step 1: Configure IPSec tunnel on R1.

R1#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#Crypto isakmp enable

R1(config)#crypto isakmp policy 10

R1(config-isakmp)#hash md5

R1(config-isakmp)#authentication pre-share

R1(config-isakmp)#group 1

R1(config-isakmp)#lifetime 84000

R1(config-isakmp)#encryption aes

R1(config-isakmp)#crypto isakmp key cisco address 22.22.22.3

R1(config)#crypto ipsec transform-set R2-Tran esp-aes esp-sha-hmac

R1(cfg-crypto-trans)#access-list 101 permit ip host 100.0.0.1 host 200.0.0.3

R1(config)#crypto map R1-R2-Map 10 ipsec-isakmp

% NOTE: This new crypto map will remain disabled until a peer

and a valid access list have been configured.

R1(config-crypto-map)#match address 101

R1(config-crypto-map)#set peer  22.22.22.3

R1(config-crypto-map)#set transform-set R2-Tran

R1(config-crypto-map)#interface g0/0

R1(config-if)#ip address 11.11.11.1 255.255.255.0

R1(config-if)#crypto map R1-R2-Map

R1(config-if)#

*May 11 13:33:26.797: %CRYPTO-6-ISAKMP_ON_OFF: ISAKMP is ON

R1(config-if)#

Step 2: Configure IPSec tunnel on R3 router.

R3#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

R3(config)#Crypto isakmp enable

R3(config)#crypto isakmp policy 10

R3(config-isakmp)#hash md5

R3(config-isakmp)#authentication pre-share

R3(config-isakmp)#group 1

R3(config-isakmp)#lifetime 84000

R3(config-isakmp)#encryption aes

R3(config-isakmp)#crypto isakmp key cisco address 11.11.11.1

R3(config)#crypto ipsec transform-set R1-Tran esp-aes esp-sha-hmac

R3(cfg-crypto-trans)#access-list 101 permit ip host 200.0.0.3 host 100.0.0.1

R3(config)#crypto map R2-R1-Map 10 ipsec-isakmp

% NOTE: This new crypto map will remain disabled until a peer

and a valid access list have been configured.

R3(config-crypto-map)#match address 101

R3(config-crypto-map)#set peer  11.11.11.1

R3(config-crypto-map)#set transform-set R1-Tran

R3(config-crypto-map)#interface gi0/0

R3(config-if)#ip address 22.22.22.3 255.255.255.0

R3(config-if)#crypto map R2-R1-Map

R3(config-if)#

*May 11 13:37:07.522: %CRYPTO-6-ISAKMP_ON_OFF: ISAKMP is ON

R3(config-if)#

Step 3: Verify IPSec tunnel by initiating ping from Loopback of R1 to loopback for R3.

R1#ping 200.0.0.3 source loopback 100

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 200.0.0.3, timeout is 2 seconds:

Packet sent with a source address of 100.0.0.1

.!!!!

Success rate is 80 percent (4/5), round-trip min/avg/max = 15/17/19 ms

Step 4: Verify if IPSec tunnel is performing encryption and encapsulation

R1#sh crypto ipsec sa

------------------------Output Trimmed---------------------------------

#pkts encaps: 4, #pkts encrypt: 4, #pkts digest: 4

#pkts decaps: 4, #pkts decrypt: 4, #pkts verify: 4

#pkts compressed: 0, #pkts decompressed: 0

#pkts not compressed: 0, #pkts compr. failed: 0

#pkts not decompressed: 0, #pkts decompress failed: 0

#send errors 0, #recv errors 0

local crypto endpt.: 11.11.11.1, remote crypto endpt.: 22.22.22.3

R1#

The above output shows that the 4-ping packets that went from R1 to R3 where encapsulated, encrypted, and hashed. Also, while receiving reply for these packets, they were also decapsulated, decrypted, and hash was verified before accepting them.